## NETWORKING ROUTING, SWITCHING & WIRELESS
PRACTICAL REPORT

Cisco Packet Tracer / CCNA-Level Laboratory Work

Prepared as a complete theory + practical + observation report

## Table of Contents

## 1. Introduction and Lab Methodology

2. Day 1 – Static Routing Concepts

3. Introduction to Dynamic Routing

4. Day 3 – Configuring RIP

5. Day 4 – Introduction to OSPF

6. Day 5 – Configuring OSPF and Failure Reconvergence

7. Switching Basics

8. VLAN Concepts

9. VLAN Trunking and Access Ports

10. Inter-VLAN Routing Basics

11. Weekly Review and Practice

12. Day 6 – Wireless Standards and Configuration

13. Troubleshooting Checklist

14. Overall Learning Outcomes

15. References

## 1. Introduction and Lab Methodology

Objective: Build a practical understanding of IP routing, dynamic routing protocols, Layer-2 switching, VLANs, inter-VLAN routing, and basic wireless security.

This report documents a sequence of networking laboratory activities using Cisco Packet Tracer concepts and Cisco IOS-style configuration. The report is written so that each activity can be reproduced from a clean topology. Commands are presented as examples for common Cisco IOS devices; interface names can differ between router and switch models in Packet Tracer.

## General laboratory method

Create the topology and connect devices using the appropriate cable or Packet Tracer automatic connection option.

Assign an IP address, subnet mask, and default gateway to end devices.

Configure router or Layer-3 switch interfaces and use no shutdown where required.

Configure the routing or switching feature requested by the activity.

Verify configuration with show commands and test end-to-end connectivity using ping.

Use Packet Tracer Simulation Mode when the task asks for protocol packets, updates, or convergence behavior.

Record observations, identify any errors, correct them, and then document the final working configuration.

Important: The exact output shown by Packet Tracer depends on the device model, IOS version, interface naming, timers, and topology. The expected observations below should be compared with the actual output from the student's own lab.

## Useful verification commands

show ip interface brief

show running-config

show ip route

show arp

show mac address-table

show vlan brief

show interfaces trunk

show cdp neighbors

ping <destination-ip>

traceroute <destination-ip>

# 2. Day 1 – Static Routing Concepts

Objective: Understand how manually configured routes allow routers to reach networks that are not directly connected.

## 2.1 Theory – What is static routing?

Static routing is a routing method in which a network administrator manually enters a route into a router's routing table. A static route tells the router how to reach a destination network by specifying a next-hop IP address, an exit interface, or both. Unlike a dynamic routing protocol, a normal static route does not automatically learn or recalculate paths when the network topology changes.

A typical IPv4 static route has the form: destination network + subnet mask + next-hop address. For example, if R1 needs to reach 192.168.2.0/24 through R2 at 10.0.0.2, R1 can be configured with:

R1(config)# ip route 192.168.2.0 255.255.255.0 10.0.0.2

## Advantages

Simple and predictable for small, stable networks.

No routing-protocol update traffic is required.

The administrator has direct control over the path.

Static routes can be useful as backup or floating routes when designed with an appropriate administrative distance.

They can provide a default route or a specific route toward a stub network.

## Disadvantages

Configuration becomes difficult to maintain as the number of networks and routers increases.

A route does not automatically adapt to a failed link unless a backup mechanism is separately configured.

Manual configuration can introduce errors such as wrong masks, next hops, or destination networks.

Network changes require an administrator to update affected routers.

Large networks are generally better suited to dynamic routing protocols.

## 2.2 Practical – Static routing between two routers

Suggested topology: PC-A — R1 — R2 — PC-B.

| Device | Interface | IP Address | Mask | Purpose |
| --- | --- | --- | --- | --- |
| PC-A | NIC | 192.168.1.10 | 255.255.255.0 | LAN A |
| R1 | G0/0 | 192.168.1.1 | 255.255.255.0 | Gateway A |
| R1 | G0/1 | 10.0.0.1 | 255.255.255.252 | Router link |
| R2 | G0/1 | 10.0.0.2 | 255.255.255.252 | Router link |
| R2 | G0/0 | 192.168.2.1 | 255.255.255.0 | Gateway B |
| PC-B | NIC | 192.168.2.10 | 255.255.255.0 | LAN B |

### R1 configuration

enable

configure terminal

interface g0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface g0/1

ip address 10.0.0.1 255.255.255.252

no shutdown

exit

ip route 192.168.2.0 255.255.255.0 10.0.0.2

end

write memory

### R2 configuration

enable

configure terminal

interface g0/0

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

interface g0/1

ip address 10.0.0.2 255.255.255.252

no shutdown

exit

ip route 192.168.1.0 255.255.255.0 10.0.0.1

end

write memory

### End-device configuration

PC-A: IP 192.168.1.10, mask 255.255.255.0, gateway 192.168.1.1.

PC-B: IP 192.168.2.10, mask 255.255.255.0, gateway 192.168.2.1.

### Verification and expected result

R1# show ip route

R1# ping 10.0.0.2

R1# ping 192.168.2.10

R2# show ip route

R2# ping 192.168.1.10

Expected observation: each router should show its directly connected networks and the manually configured remote LAN with an 'S' route in the IPv4 routing table. After the PCs have correct gateways and both routers have the required routes, an end-to-end ping from PC-A to PC-B should succeed.

Analysis: Static routing works because both routers have explicit knowledge of the remote LAN. If either static route is missing, the return path may fail even when the forward path is correct.

# 3. Introduction to Dynamic Routing

Objective: Understand why routers use routing protocols to exchange reachability information and automatically adapt to topology changes.

## 3.1 What is dynamic routing?

Dynamic routing is the process in which routers use a routing protocol to exchange information about reachable networks and calculate routes automatically. Examples include RIP, OSPF, EIGRP, and BGP. In an enterprise LAN, OSPF is commonly used as an interior gateway protocol, while RIP is mainly useful for learning routing fundamentals and for small or legacy environments.

## 3.2 Static versus dynamic routing

| Feature | Static Routing | Dynamic Routing |
| --- | --- | --- |
| Configuration | Manually configured | Learned through routing protocols |
| Topology changes | Requires manual action in normal use | Can automatically recalculate paths |
| Control | Very direct and predictable | Protocol determines best path |
| Overhead | Very low protocol overhead | Uses CPU, memory and routing messages |
| Scalability | Good for small/stable networks | Better for larger changing networks |
| Failure recovery | Needs backup design/manual change | Can reconverge automatically |
| Examples | ip route | RIP, OSPF, EIGRP, BGP |

## 3.3 Practical – Observe RIP updates in a pre-configured network

Open the supplied/pre-configured Packet Tracer topology containing at least two routers running RIP.

Confirm the router interfaces are up with show ip interface brief.

Open Simulation Mode and filter for RIP if the Packet Tracer version provides the protocol filter.

Generate traffic or allow the topology to exchange routing information.

Inspect the packets sent between routers. RIP commonly uses periodic routing updates rather than building a complete link-state database.

On a router, use show ip route and identify routes learned through RIP. Cisco IOS commonly marks RIP-learned IPv4 routes with the letter R.

Record which destination networks appear after RIP learns them and compare this with the topology.

show ip protocols

show ip route

show ip route rip

Observation to record: RIP routers exchange reachability information with neighboring routers. After updates are processed, remote networks can appear in the routing table without manually entering a separate static route for each network.

# 4. Day 3 – Configuring RIP

Objective: Configure RIP in a multi-router topology, understand hop count and updates, and analyze routing changes in Simulation Mode.

## 4.1 RIP concepts

RIP (Routing Information Protocol) is a distance-vector interior gateway protocol. The classic RIP metric is hop count: each router crossed toward a destination increases the metric by one. A route with a lower hop count is preferred, subject to the protocol's rules. RIP uses a maximum usable metric of 15 hops; a metric of 16 represents an unreachable destination. This limitation is one reason RIP is not suitable for large modern enterprise networks.

### RIP updates

RIP routers periodically exchange routing information with neighbors. RIPv2 supports classless routing and carries subnet-mask information, which allows variable-length subnetting and discontiguous networks to be represented more accurately than classful RIPv1.

### Convergence

Convergence is the period during which routers detect a topology change, exchange the new information, and reach a consistent view of routes. RIP can converge more slowly than link-state protocols such as OSPF, especially in larger or more complex topologies.

## 4.2 Practical – Configure RIPv2

Suggested topology: R1 — R2 — R3, with one LAN attached to each router.

| Router | LAN | Transit Link |
| --- | --- | --- |
| R1 | 192.168.10.0/24 | 10.0.12.0/30 |
| R2 | 192.168.20.0/24 | 10.0.12.0/30 and 10.0.23.0/30 |
| R3 | 192.168.30.0/24 | 10.0.23.0/30 |

### Example R1

enable

configure terminal

router rip

version 2

no auto-summary

network 192.168.10.0

network 10.0.0.0

end

### Example R2

enable

configure terminal

router rip

version 2

no auto-summary

network 192.168.20.0

network 10.0.0.0

end

### Example R3

enable

configure terminal

router rip

version 2

no auto-summary

network 192.168.30.0

network 10.0.0.0

end

The network statements must match the actual connected networks/interfaces in the chosen Packet Tracer topology. Some Cisco IOS versions accept a major network statement that activates RIP on matching interfaces, so always verify the result instead of assuming the command behaved exactly as intended.

## 4.3 Capture and analyze RIP updates

Switch Packet Tracer to Simulation Mode.

Choose Edit Filters and display RIP packets if available.

Run the simulation or use Capture/Forward to step through packets.

Identify the router sending a routing update and the neighbor receiving it.

Inspect the destination networks and hop-count information carried in the update.

Compare the routing table before and after the update.

Disconnect or shut down one router interface and observe how routing information changes after the network detects the failure.

R1# show ip protocols

R1# show ip route

R1# show ip route rip

R1# show running-config | section router rip

Expected analysis: RIP learns remote networks from neighbors and installs valid routes. A topology change causes routing information to change after the protocol's update and convergence process. The exact timing depends on timers and Packet Tracer behavior.

# 5. Day 4 – Introduction to OSPF

Objective: Understand OSPF areas, link-state advertisements, cost, neighbor adjacency, and the basic Cisco configuration process.

## 5.1 How OSPF works

OSPF (Open Shortest Path First) is a link-state interior gateway protocol. Instead of simply advertising a list of destinations as a distance-vector protocol does, OSPF routers build a link-state database (LSDB) describing the topology relevant to their area. Routers then run the Shortest Path First (SPF) algorithm to calculate the best paths.

### Areas

OSPF uses areas to create a hierarchical design. Area 0 is the backbone area and is the central part of a conventional multi-area OSPF design. An Area Border Router (ABR) connects OSPF areas and exchanges summarized/inter-area information. Area design can reduce the amount of topology information that every router must process.

### LSAs

Link-State Advertisements (LSAs) are the building blocks used by OSPF to describe topology and reachability information. Common LSA types include Type 1 Router LSAs, Type 2 Network LSAs on multiaccess networks, Type 3 Summary LSAs between areas, and Type 5 AS-External LSAs for external routes. The exact set visible depends on the topology and OSPF area design.

### OSPF cost

OSPF uses cost as its primary path metric. On Cisco IOS, interface cost can be explicitly configured with ip ospf cost. If cost is not manually set, Cisco calculates it from interface bandwidth using the OSPF reference bandwidth. A path's total cost is derived from the costs of the links along that path.

## 5.2 Observe OSPF adjacency formation

Create or open a topology with two or more routers connected to the same OSPF area.

Assign IP addresses and bring interfaces up.

Enable OSPF on the required interfaces using the Packet Tracer-supported method.

Use show ip ospf neighbor to observe discovered neighbors.

If the routers are on an appropriate multiaccess network, identify DR/BDR behavior where applicable.

Use Simulation Mode to observe OSPF packets such as Hello messages and database-related exchanges.

Wait for the neighbor relationship to reach FULL where a full adjacency is expected.

show ip ospf neighbor

show ip ospf interface brief

show ip ospf

show ip route ospf

show ip ospf database

## 5.3 Steps to configure OSPF

Plan the IP addressing and decide which links belong to which OSPF area.

Configure all router interfaces and verify that they are up/up.

Start an OSPF process with router ospf <process-id>.

Choose a stable router ID or allow Cisco IOS to select one according to its rules.

Advertise the intended networks or enable OSPF directly on the relevant interfaces, depending on the IOS/platform.

Verify neighbor relationships.

Verify that OSPF routes are installed.

Test end-to-end connectivity and record the results.

R1(config)# router ospf 1

R1(config-router)# router-id 1.1.1.1

R1(config-router)# network 192.168.10.0 0.0.0.255 area 0

R1(config-router)# network 10.0.12.0 0.0.0.3 area 0

# 6. Day 5 – Configuring OSPF and Failure Reconvergence

Objective: Configure a multi-area OSPF network and observe how OSPF reacts when a router or link fails.

## 6.1 Multi-area OSPF design

A simple lab can contain Area 0 as the backbone and Area 1 as a non-backbone area. The router connecting the two areas acts as an ABR. For example: R1 in Area 0, R2 as the ABR in Areas 0 and 1, and R3 in Area 1.

| Router | Role | Example Networks | Area |
| --- | --- | --- | --- |
| R1 | Backbone router | 192.168.10.0/24, 10.0.12.0/30 | 0 |
| R2 | ABR | 10.0.12.0/30, 10.0.23.0/30 | 0 and 1 |
| R3 | Area 1 router | 192.168.30.0/24, 10.0.23.0/30 | 1 |

## 6.2 Example configuration

### R1 – Area 0

router ospf 1

router-id 1.1.1.1

network 192.168.10.0 0.0.0.255 area 0

network 10.0.12.0 0.0.0.3 area 0

### R2 – ABR

router ospf 1

router-id 2.2.2.2

network 10.0.12.0 0.0.0.3 area 0

network 10.0.23.0 0.0.0.3 area 1

### R3 – Area 1

router ospf 1

router-id 3.3.3.3

network 10.0.23.0 0.0.0.3 area 1

network 192.168.30.0 0.0.0.255 area 1

Design note: The commands above are a lab example, not a universal template. The wildcard masks must correspond to the real subnet masks in the student's topology.

## 6.3 Failure and reconvergence experiment

Verify that all OSPF neighbors are established and that end-to-end pings work.

Record the routing table and OSPF neighbor table before the failure.

Shut down a selected router interface or remove a link in Packet Tracer.

Observe the neighbor relationship and routing table after the failure.

Use Simulation Mode to observe OSPF messages generated by the topology change.

If an alternate path exists, verify that traffic eventually uses the alternate path.

Restore the failed interface with no shutdown and observe the routing table return to the preferred topology.

show ip ospf neighbor

show ip route

show ip route ospf

show ip ospf database

Expected result: OSPF detects the topology change, updates link-state information, recalculates affected paths, and can install an alternate route when one exists. This process is called reconvergence.

## 6.4 Troubleshooting OSPF

Check that interfaces are up/up and IP addresses are correct.

Confirm both routers are in the same OSPF area on a shared link.

Check that the OSPF network statement/wildcard or interface activation actually matches the interface.

Check router IDs and duplicate router-ID problems.

Check hello/dead interval consistency on directly connected OSPF neighbors.

Check authentication settings if authentication is configured.

Check that the intended network is present in the OSPF database and routing table.

Use show ip ospf neighbor, show ip ospf interface, and show ip ospf database to isolate the problem.

# 7. Switching Basics

Objective: Understand how an Ethernet switch learns MAC addresses and decides where to forward frames.

## 7.1 How a switch operates

A Layer-2 Ethernet switch forwards frames using destination MAC addresses. When a frame arrives, the switch examines the source MAC address and associates it with the incoming port in its MAC address table. This is called MAC learning. The switch then examines the destination MAC address to decide whether to forward the frame to a known port, flood it within the relevant broadcast domain, or filter it.

### MAC address table

A switch's MAC address table maps learned MAC addresses to interfaces and VLANs. Entries can be dynamically learned and can age out after a period of inactivity. Static MAC entries can also be configured, depending on the design.

### Frame forwarding process

A frame enters a switch port.

The switch reads the source MAC address and learns it on the ingress port for the relevant VLAN.

The switch checks the destination MAC address in the MAC table.

If the destination is known and associated with another port in the same VLAN, the frame is forwarded only toward that port.

If the destination is unknown, the switch floods the frame out other eligible ports in the same VLAN, excluding the ingress port.

Broadcast and certain multicast traffic is replicated within the applicable broadcast domain according to switch behavior.

The destination device receives the frame and processes it if the Layer-2 and Layer-3 information is appropriate.

## 7.2 Practical – Observe MAC table population

Connect two or more PCs to a switch.

Assign IP addresses in the same subnet.

Check the MAC table before generating traffic.

Ping between PCs.

Run show mac address-table and record newly learned MAC addresses.

Repeat the test and observe that the switch already knows the destination port.

Wait for dynamic entries to age or clear them if the lab requires a fresh learning experiment.

Switch# show mac address-table

Switch# show mac address-table dynamic

Switch# clear mac address-table dynamic

Observation: The first traffic exchange may cause MAC learning. After learning, unicast frames can be forwarded directly to the port associated with the destination MAC address instead of being flooded throughout the VLAN.

# 8. VLAN Concepts

Objective: Understand network segmentation with VLANs and configure switch ports for separate broadcast domains.

## 8.1 What is a VLAN?

A VLAN (Virtual Local Area Network) is a logical Layer-2 network segment created on a switch. Devices in different VLANs are separated into different broadcast domains even when they share the same physical switch infrastructure. Communication between VLANs requires Layer-3 routing.

## Benefits and use cases

Reduces broadcast-domain size.

Separates users or departments logically.

Supports security and policy boundaries when combined with Layer-3 controls and access policies.

Allows devices to remain in the same logical network even when physical locations change, depending on the design.

Makes larger switched networks easier to organize.

## 8.2 Practical – Create VLANs and assign ports

enable

configure terminal

vlan 10

name SALES

exit

vlan 20

name IT

exit

interface range f0/1-10

switchport mode access

switchport access vlan 10

exit

interface range f0/11-20

switchport mode access

switchport access vlan 20

exit

end

### Verification

show vlan brief

show interfaces status

show running-config

Expected result: ports assigned to VLAN 10 appear under that VLAN, while ports assigned to VLAN 20 appear under VLAN 20. A host in VLAN 10 cannot directly communicate at Layer 2 with a host in VLAN 20; a Layer-3 device is required for inter-VLAN communication.

## 8.3 VLAN configuration documentation

| VLAN ID | Name | Purpose | Example Access Ports | Subnet |
| --- | --- | --- | --- | --- |
| 10 | SALES | Sales users | Fa0/1–Fa0/10 | 192.168.10.0/24 |
| 20 | IT | IT users | Fa0/11–Fa0/20 | 192.168.20.0/24 |
| 99 | MGMT | Management (optional) | As designed | 192.168.99.0/24 |

# 9. VLAN Trunking and Access Ports

Objective: Understand the difference between access and trunk ports and carry multiple VLANs across a single link.

## 9.1 Access ports

An access port is normally assigned to one VLAN and is commonly used for end devices such as PCs, printers, and some non-trunking devices. Frames sent to an access device are normally untagged on the endpoint-facing link.

## 9.2 Trunk ports

A trunk link carries traffic for multiple VLANs between network devices such as switches, routers, and Layer-3 switches. IEEE 802.1Q is the widely used VLAN tagging standard. The tag identifies the VLAN associated with the frame while it traverses the trunk.

ISL (Inter-Switch Link) was a Cisco proprietary VLAN trunking protocol used on older equipment. It is largely historical in modern Ethernet networks, where IEEE 802.1Q is the standard approach.

## 9.3 Practical – Configure a trunk

enable

configure terminal

interface f0/24

switchport mode trunk

switchport trunk allowed vlan 10,20

end

On some Packet Tracer switch models or IOS versions, trunk encapsulation commands may not be available because 802.1Q is the only supported encapsulation. Do not add an unsupported encapsulation command.

show interfaces trunk

show interfaces f0/24 switchport

## 9.4 Simulate VLAN communication over a trunk

Create VLAN 10 and VLAN 20 on both switches.

Assign end-device ports to the correct VLANs.

Configure the inter-switch link as a trunk.

Allow VLAN 10 and VLAN 20 on the trunk.

Verify both switches show the trunk as operational.

Test same-VLAN communication across the two switches.

Test different-VLAN communication; it should fail until Layer-3 inter-VLAN routing is configured.

Key point: A trunk carries multiple VLANs; it does not by itself route between VLANs. Routing is a Layer-3 function.

# 10. Inter-VLAN Routing Basics

Objective: Configure router-on-a-stick and understand the role of Layer-3 switches in routing between VLANs.

## 10.1 Why inter-VLAN routing is needed

VLANs create separate broadcast domains. If VLAN 10 and VLAN 20 use different IP subnets, a device in VLAN 10 needs a Layer-3 gateway to communicate with a device in VLAN 20. That gateway can be a router or a multilayer switch.

## 10.2 Router-on-a-stick

Router-on-a-stick uses one physical router interface configured as a trunk connection to a switch. The router creates multiple logical subinterfaces, one for each VLAN, and each subinterface acts as the default gateway for its VLAN.

| VLAN | Subnet | Router Gateway | Switch Port |
| --- | --- | --- | --- |
| 10 | 192.168.10.0/24 | 192.168.10.1 | Access ports in VLAN 10 |
| 20 | 192.168.20.0/24 | 192.168.20.1 | Access ports in VLAN 20 |

### Switch configuration

vlan 10

name SALES

vlan 20

name IT

interface f0/1

switchport mode access

switchport access vlan 10

interface f0/2

switchport mode access

switchport access vlan 20

interface f0/24

switchport mode trunk

switchport trunk allowed vlan 10,20

### Router configuration

interface g0/0

no shutdown

exit

interface g0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

exit

interface g0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

exit

### End-device gateways

VLAN 10 hosts: default gateway 192.168.10.1.

VLAN 20 hosts: default gateway 192.168.20.1.

### Verification

show vlan brief

show interfaces trunk

show ip interface brief

show ip route

ping 192.168.10.1

ping 192.168.20.1

## 10.3 Layer-3 switch method

A multilayer switch can perform inter-VLAN routing using switched virtual interfaces (SVIs). Each VLAN gets an interface such as interface vlan 10 with an IP address, and IP routing is enabled on the switch.

vlan 10

name SALES

vlan 20

name IT

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

interface vlan 20

ip address 192.168.20.1 255.255.255.0

no shutdown

ip routing

Comparison: Router-on-a-stick is easy to understand and suitable for many labs, but all inter-VLAN traffic uses the router link. A multilayer switch can route between VLANs in hardware and is generally more scalable for campus LAN designs.

## 10.4 Troubleshooting inter-VLAN routing

Confirm both VLANs exist on the switch.

Confirm each host is connected to the correct access VLAN.

Confirm the switch-to-router link is actually a trunk.

Confirm the required VLANs are allowed on the trunk.

Confirm router subinterfaces use the correct 802.1Q VLAN IDs.

Confirm each subinterface has the correct gateway IP and mask.

Confirm the router physical interface is not administratively down.

Confirm host default gateways point to the correct VLAN gateway.

Use ping from the host to its own gateway, then to the other VLAN gateway, then to the remote host.

Use show vlan brief, show interfaces trunk, show ip interface brief, and show ip route to isolate the failure.

# 11. Weekly Review and Practice

Objective: Combine VLAN creation, trunking, and inter-VLAN routing into one integrated topology and document the result.

## 11.1 Integrated topology

Recommended lab: two switches, one router or multilayer switch, at least four PCs, VLAN 10 for one group and VLAN 20 for another group. The switches are connected by a trunk, and the routing device provides gateways for both VLANs.

Create VLAN 10 and VLAN 20 on both switches.

Assign at least two access ports to VLAN 10 and two access ports to VLAN 20.

Configure the inter-switch link as a trunk and allow VLANs 10 and 20.

Configure router-on-a-stick or SVIs for the two VLANs.

Configure the correct IP address and gateway on every PC.

Verify VLAN membership and trunk status.

Ping within the same VLAN and across VLANs.

Record the commands used and the successful verification results.

Create a short troubleshooting section describing at least one problem and how it was solved.

## 11.2 Sample report observation

After the VLANs and trunk were configured, devices in the same VLAN could communicate across the switched infrastructure. Initially, devices in different VLANs could not communicate because VLAN separation creates separate broadcast domains. After configuring the router subinterfaces and assigning the correct default gateways, inter-VLAN pings succeeded. This demonstrated that trunking transports VLAN traffic while Layer-3 routing provides communication between the VLAN subnets.

## 11.3 Suggested final verification table

| Test | Expected Result | Actual Result |
| --- | --- | --- |
| PC in VLAN 10 → VLAN 10 gateway | Success | ________ |
| PC in VLAN 20 → VLAN 20 gateway | Success | ________ |
| VLAN 10 PC → VLAN 20 PC | Success after routing | ________ |
| Switch 1 → Switch 2 trunk | Operational | ________ |
| show vlan brief | Correct VLAN/port membership | ________ |
| show interfaces trunk | Correct trunk and allowed VLANs | ________ |

# 12. Day 6 – Wireless Standards and Configuration

Objective: Understand basic Wi-Fi standards, channel selection, interference avoidance, wireless security, MAC filtering, and access-control concepts.

## 12.1 Wireless setup

A basic wireless LAN normally contains an access point or wireless router, one or more wireless clients, and a wired network connection when Internet or LAN access is required. A practical setup should define the SSID, radio settings, addressing method, security method, and management credentials.

## 12.2 Wireless standards

| Standard / Technology | Main idea | Typical relevance |
| --- | --- | --- |
| 802.11b | Legacy 2.4 GHz Wi-Fi, lower data rates | Historical/legacy |
| 802.11g | 2.4 GHz improvement over 802.11b | Legacy |
| 802.11n (Wi-Fi 4) | Introduced improvements such as MIMO and operation in 2.4/5 GHz | Widely deployed historically |
| 802.11ac (Wi-Fi 5) | Primarily 5 GHz, wider channels and higher throughput | Common modern legacy |
| 802.11ax (Wi-Fi 6/6E) | Efficiency improvements, OFDMA and better dense-network performance | Modern deployments |

Exact supported bands, channel widths, and features depend on the access point, client, regulatory domain, and configuration. The lab should use the options available in the selected Packet Tracer device.

## 12.3 Channel selection and avoiding interference

Wireless performance can be affected by co-channel interference, adjacent-channel interference, physical obstacles, competing access points, Bluetooth and other radio sources, and poor channel planning. In the 2.4 GHz band, the commonly used non-overlapping 20 MHz channel plan is based on channels 1, 6, and 11 in many regulatory environments. Channel availability differs by country and band, so real deployments must follow local regulations and access-point recommendations.

Prefer a clean channel based on an RF scan or site survey rather than choosing a channel blindly.

Avoid placing neighboring access points on overlapping channels when they are close together.

Use 5 GHz or 6 GHz where supported when the deployment benefits from additional spectrum and shorter-range high-throughput connectivity.

Keep access points away from major physical obstructions and strong sources of electromagnetic interference.

Use appropriate transmit power so nearby access points do not create unnecessary interference.

## 12.4 Wireless security

Wireless security protects the confidentiality and integrity of WLAN traffic and controls which users/devices can connect. Modern deployments should prefer WPA2 or WPA3 depending on device capabilities and organizational requirements. WEP is obsolete and should not be used for a modern network.

Use WPA3 when supported and appropriate; WPA2 remains common for compatibility.

Use a strong, unique passphrase for personal/PSK networks.

For enterprise environments, 802.1X with an authentication server can provide per-user authentication and stronger centralized access control.

Change default management credentials and restrict management access.

Keep access-point firmware/software updated according to the vendor's supported process.

Separate guest access from internal resources where possible.

## 12.5 MAC filtering

MAC filtering allows a WLAN administrator to permit or deny devices based on their MAC addresses. It can be useful as an administrative access-control layer, but it should not be treated as strong authentication by itself because MAC addresses can be spoofed. For this reason, MAC filtering should supplement, not replace, proper wireless authentication and encryption.

## 12.6 Access Control Lists (ACLs)

ACLs are ordered rules that permit or deny traffic according to criteria such as source/destination IP address, protocol, ports, or—in some Cisco wireless contexts—MAC addresses. An ACL should be designed carefully because rule order matters and an implicit deny behavior can block traffic that was not explicitly permitted.

## 12.7 Practical – Basic wireless setup

Place a wireless router/access point in Packet Tracer.

Configure a clear SSID such as CAMPUS-WIFI.

Choose the supported wireless standard/band in the simulator.

Select an appropriate channel and avoid unnecessary overlap with nearby APs.

Configure WPA2/WPA3 security if the simulated device supports it.

Create a strong wireless password for the lab.

Connect a wireless client and enter the correct SSID and security credentials.

Verify that the client receives an IP address or configure the appropriate static address.

Ping the default gateway and then test access to another permitted host.

If the task includes MAC filtering, add the approved client MAC address and verify that an unapproved client is denied.

## 12.8 Wireless troubleshooting checklist

SSID is correct and visible.

Client is within usable radio range.

Client supports the selected band/security method.

Wireless password/security settings match.

DHCP service is available if automatic addressing is expected.

Default gateway is correct.

MAC filtering or ACL rules are not unintentionally blocking the client.

Channel selection is reasonable and does not create unnecessary interference.

The access point has a working connection to the wired LAN if LAN/Internet access is required.

# 13. Troubleshooting Checklist

Objective: Provide a repeatable method for diagnosing common Packet Tracer routing and switching failures.

| Problem | Likely Causes | Useful Checks |
| --- | --- | --- |
| Ping fails | Wrong IP/mask/gateway, interface down, missing route | ipconfig, show ip int brief, show ip route |
| Static route missing | Wrong destination/mask/next hop | show ip route, show run |
| RIP route absent | Wrong network statement, interface down, RIP mismatch | show ip protocols, show ip route |
| OSPF neighbor not forming | Area mismatch, IP/subnet issue, timers/authentication | show ip ospf neighbor, show ip ospf interface |
| VLAN host isolated | Wrong access VLAN or VLAN missing | show vlan brief |
| Trunk not working | Port not trunking, VLAN not allowed | show interfaces trunk |
| Inter-VLAN ping fails | Wrong gateway/subinterface/tagging | show ip int brief, show run, show interfaces trunk |
| Wireless client cannot join | SSID/security/band/MAC filter issue | AP settings and client configuration |

## A logical troubleshooting sequence

Check physical/link status first.

Check addressing and subnet masks.

Check the local default gateway.

Check Layer-2 VLAN and trunk configuration.

Check routing table and route selection.

Check the protocol-specific neighbor/operation state.

Test hop-by-hop instead of only testing the final destination.

Change one configuration item at a time and retest.

Document the cause, fix, and verification result.

# 14. Overall Learning Outcomes

Objective: Summarize the technical skills demonstrated across the complete laboratory sequence.

Explain the purpose and limitations of static routing.

Configure and verify static routes between routers.

Explain why dynamic routing is useful in changing networks.

Understand RIP hop count, routing updates, and convergence behavior.

Configure RIPv2 and inspect learned routes.

Explain OSPF areas, LSAs, cost, neighbor adjacency, and SPF-based path calculation.

Configure basic single-area and multi-area OSPF in a lab environment.

Observe OSPF reconvergence after a simulated failure.

Explain how switches learn MAC addresses and forward Ethernet frames.

Create VLANs and assign switch access ports.

Configure and verify VLAN trunks using IEEE 802.1Q.

Configure router-on-a-stick inter-VLAN routing.

Understand the role of Layer-3 switches and SVIs.

Troubleshoot common VLAN, trunk, routing, and gateway problems.

Understand wireless standards, channel planning, interference avoidance, security, MAC filtering, and ACL concepts.

## Final conclusion

The laboratory sequence demonstrates how modern computer networks are built in layers. Static routing provides direct manual control for simple or stable paths, while dynamic protocols such as RIP and OSPF automate route learning and adaptation. At Layer 2, switches use MAC addresses to forward Ethernet frames and VLANs to create logical broadcast domains. Trunks extend multiple VLANs between network devices, while inter-VLAN routing provides the Layer-3 communication required between separate subnets. Wireless networking adds radio-specific design considerations such as channel selection, interference, authentication, encryption, and client access control. Together, these activities provide a practical foundation for configuring, verifying, and troubleshooting small-to-medium network topologies in Cisco Packet Tracer.

# 15. References

Objective: Authoritative and supporting sources used for the technical explanations and command examples.

Cisco Systems – Configuring Static Routes – https://www.cisco.com/c/en/us/td/docs/routers/ncs5xx/ncs520/configuration/guide/iproute/17-1-1/b-ipv4-routing-xe-17-1-1-ncs520/b-ipv4-routing-xe-17-1-1-ncs520_chapter_0101.html

Cisco Systems – Configuring OSPF – https://www.cisco.com/c/en/us/td/docs/routers/ios-xe/ip-routing/b-ip-routing/m_iro-cfg-0.html

Cisco Systems – MAC Filtering on Wireless LAN Controllers – https://www.cisco.com/c/en/us/support/docs/wireless-mobility/wlan-security/91901-mac-filters-wlcs-config.html

Cisco Systems – Wireless Access Point ACL Filter Configuration – https://www.cisco.com/c/en/us/support/docs/wireless-mobility/wireless-lan-wlan/68097-accesspt.html

Cisco Systems – Securing a Wireless LAN – https://www.cisco.com/c/en/us/td/docs/ios/12_4t/wlan/configuration/guide/wlcgsec.html

IETF RFC 2328 – OSPF Version 2 – https://www.rfc-editor.org/rfc/rfc2328

IETF RFC 2453 – RIP Version 2 – https://www.rfc-editor.org/rfc/rfc2453

IEEE 802.1Q – VLAN/Bridging standard family – https://standards.ieee.org/ieee/802.1Q/

## Submission note

Before submitting this report, replace the blank student-information fields and, where required by your instructor, add screenshots from your own Packet Tracer topology. Screenshots are especially useful for showing show ip route, show ip ospf neighbor, show vlan brief, show interfaces trunk, successful pings, and Simulation Mode observations.